NAME:B.Tejaswini

REGNO.:192412684

ECA1803 – OPTOELECTRONICS AND OPTICAL COMMUNICATION

CO5 – CASE STUDY BASED ASSIGNMENT

DWDM Metro Link Commissioning:
From Field Measurements to a Standards-Based Upgrade Decision

Executive Summary

This case study evaluates a 60 km G.652.D metro fiber link carrying eight 10 Gb/s NRZ DWDM channels on a 100 GHz grid. Field data indicate excessive loss at a mechanical splice and a poor connector near the far end, while Channel 5 has a low extinction ratio, high jitter and a partially closed eye. The report calculates the link budget, OTDR limitations, BER/Q performance, quantum detection limits and energy/spectral trade-offs for three 1.6 Tb/s upgrade options.

Recommended action: repair the 1.20 dB reflective connector event first, replace the 0.62 dB mechanical splice with a fusion splice, and retest from both ends using long-pulse and short-pulse OTDR settings as appropriate. Channel 5 must be repaired or replaced because its ER and Q fail the stated acceptance criteria. For the upgrade, Option Q (8 × 200G DP-16QAM) is the balanced recommendation: it reaches 1.6 Tb/s using about 160 W of module power, occupies about 300 GHz under the supplied flex-grid assumptions, and has a calculated OSNR margin of 27.5 dB using the brief's OSNR relation. Option R uses less power but needs 26 dB OSNR and has less OSNR margin.

## Case Parameters

| Parameter | Given value |
| --- | --- |
| Fiber | 60 km, ITU-T G.652.D; attenuation 0.21 dB/km at 1550 nm |
| Existing WDM | 8 × 10 Gb/s NRZ, 100 GHz grid, +1 dBm/channel at line output |
| Passive losses | Demultiplexer 6.0 dB; 3 measured connectors at 0.5 dB each; 12 fusion splices at 0.05 dB each; event 2 mechanical splice 0.62 dB; event 3 connector 1.20 dB |
| Receiver | Sensitivity −24 dBm at BER 10⁻¹² if ER ≥ 8.2 dB; required margin ≥ 3 dB |
| Amplifier | EDFA NF 5 dB; input −6 dBm/channel; required OSNR 20 dB in 0.1 nm |
| Performance | Channel 3: Q=7.6, ER=9.0 dB, DJ=0.18 UI, RJ=0.012 UI rms; Channel 5: Q=4.7, ER=4.5 dB, DJ=0.30 UI, RJ=0.022 UI rms |
| Upgrade target | 1.6 Tb/s on same fiber pair; compare options P, Q and R |

# Stage 1 – Interpret the Measurement and Performance Requirement (10 Marks)

## 1.1 Instruments and validated quantities

| Instrument | Measurement / use |
| --- | --- |
| Optical power meter | Measures launch/received optical power and insertion loss; validates receiver margin. |
| Optical spectrum analyzer (OSA) | Measures wavelength, channel power, spectral shape, OSNR and channel-to-channel flatness. |
| OTDR | Locates events by distance and estimates loss/reflectance; dead zones limit closely spaced event analysis. |
| Eye / BER tester | Eye opening, extinction ratio, deterministic/random jitter, Q-factor and BER; identifies channel-specific degradation. |
| Dispersion / PMD analyzer | Measures chromatic dispersion and polarization-mode-dispersion characteristics. |

Acceptance targets are: receiver power at least −21 dBm if the −24 dBm sensitivity must include 3 dB margin; BER ≤ 10⁻¹²; Q ≥ 7.03; ER ≥ 8.2 dB; OSNR ≥ 20 dB in 0.1 nm for the legacy link; and no unexplained excess-loss events. ITU-T G.652.D defines the single-mode fiber characteristics and ITU-T G.694.1 defines DWDM frequency-grid planning.

## 1.2 OTDR settings and limitations

The OTDR's event dead zone (EDZ) governs how close a reflective event can be to another event and still be separately identified; attenuation dead zone (ADZ) governs how soon the backscatter trace can be accurately measured after an event. Short pulses improve spatial resolution but reduce dynamic range. The 9 m spacing between events 3 and 4 is resolvable with the 10 ns setting (EDZ 1 m, ADZ 4 m), but that setting has only 20 dB dynamic range. The 100 ns setting has 27 dB dynamic range but a 15 m ADZ, so it cannot reliably separate or measure the second connector 9 m after event 3.

Figure 1. Conceptual OTDR trace showing the supplied event locations; trace is illustrative, not measured data.

Use a higher-dynamic-range, longer-pulse trace from the far end to characterize the complete span, then a short-pulse localized test from the end closest to events 3 and 4. Use bidirectional traces to reduce uncertainty in event-loss interpretation. Spatial distance represented by the OTDR round-trip measurement is approximately cτ/(2n): 10 ns ≈ 1.02 m, 100 ns ≈ 10.21 m, and 1 μs ≈ 102.1 m for n=1.4682.

## 1.3 Performance requirement statement

Repair the existing 60 km link without replacing the fiber pair; achieve ≤21.25 dB total design loss, received power with ≥3 dB margin, BER ≤10⁻¹² and Q ≥7.03, and meet the OSNR target. Then upgrade to 1.6 Tb/s while meeting the chosen channel-grid, spectral-efficiency, power and interoperability constraints.

# Stage 2 – Link Budget, Dispersion and OSNR (20 Marks)

## 2.1 As-built link loss

| Component | Calculation | Loss (dB) |
| --- | --- | --- |
| Fiber attenuation | 60 km × 0.21 dB/km | 12.60 |
| Measured connectors | 3 × 0.50 dB | 1.50 |
| Fusion splices | 12 × 0.05 dB | 0.60 |
| Mechanical splice (event 2) | Measured OTDR event | 0.62 |
| Poor connector (event 3) | Measured OTDR event | 1.20 |
| Demultiplexer | Given | 6.00 |
| As-built total | Sum of above | 22.52 |
| Design allowance total | 12.60 + 4×0.5 + 13×0.05 + 6.0 | 21.25 |
| Excess above design | 22.52 − 21.25 | 1.27 |

The 1.20 dB connector event is the largest individual excess-loss event and should be the first repair priority. The 0.62 dB mechanical splice is also unacceptable compared with a typical fusion-splice allowance of 0.05 dB.

## 2.2 Received power and penalties

Received power before dispersion/extinction-ratio penalties: P_rx = +1 dBm − 22.52 dB = −21.52 dBm/channel. The raw sensitivity margin is −21.52 − (−24) = 2.48 dB, which is 0.52 dB below the required 3 dB margin even before penalties.

Using the provided dispersion relation δ = 5 log₁₀[1 + 2π(BDLσλ)²], with B=10 Gb/s, D=17 ps/(nm·km), L=60 km and σλ=0.03 nm gives BDLσλ = 0.306 and δ ≈ 3.36 dB. This is a simplified model; actual dispersion penalty depends on transmitter chirp, receiver filtering and the measurement definition.

For the common extinction-ratio penalty approximation, penalty ≈ −10 log₁₀[(ERlinear−1)/(ERlinear+1)]: Channel 3, ER=9 dB (ERlinear≈7.94), penalty ≈1.13 dB; Channel 5, ER=4.5 dB (ERlinear≈2.82), penalty ≈3.28 dB.

| Quantity | Channel 3 | Channel 5 |
| --- | --- | --- |
| Received power before penalties | −21.52 dBm | −21.52 dBm |
| Dispersion penalty (model) | 3.36 dB | 3.36 dB |
| ER penalty (approx.) | 1.13 dB | 3.28 dB |
| Effective power after penalties | −26.01 dBm | −28.16 dBm |
| Raw margin vs −24 dBm | 2.48 dB | 2.48 dB |
| Q acceptance (≥7.03) | Pass (7.6) | Fail (4.7) |
| ER acceptance (≥8.2 dB) | Pass (9.0 dB) | Fail (4.5 dB) |

The penalties are quality penalties and should not be interpreted as a literal optical power-meter reading. Channel 3 is only marginal on raw power and has limited remaining penalty budget; Channel 5 is unacceptable due to its poor ER, elevated jitter and low Q. Correct the physical losses and restore Channel 5 transmitter/driver bias or replace the transmitter before increasing launch power.

## 2.3 OSNR and PMD

Using the relation specified in the brief, OSNR = 58 + P_in − NF = 58 + (−6) − 5 = 47 dB in 0.1 nm. This exceeds the stated 20 dB requirement by 27 dB. Under the supplied model, ASE/OSNR is not the primary legacy-link limitation; excess loss and Channel 5's transmitter/eye quality dominate. This result should be checked against a measured OSA OSNR because the formula is a simplified budget relation.

PMD DGD estimate = PMD coefficient × √L = 0.1 ps/√km × √60 = 0.775 ps. The 10 Gb/s bit period is 100 ps, so 10% is 10 ps. The estimated PMD is well below the tolerance.

## Design Decision 1

I recommend repairing the 1.20 dB connector and 0.62 dB mechanical splice before any launch-power increase because the as-built loss is 1.27 dB above the design budget and the raw received-power margin is already below 3 dB. The calculated OSNR is ample under the provided relation, so power/penalties—not ASE noise—are the immediate priority.

# Stage 3 – OTDR, Splice and Connector Strategy (20 Marks)

| Event | Reading | Interpretation / action |
| --- | --- | --- |
| Event 1, 12.00 km | 0.05 dB, non-reflective | Consistent with a low-loss fusion splice; retain and document. |
| Event 2, 31.50 km | 0.62 dB, reflectance −34 dB | High-loss mechanical splice; replace with fusion splice, then bidirectional OTDR test. |
| Event 3, 59.900 km | 1.20 dB, reflectance −22 dB | Poor LC/UPC patch-panel connector; inspect/clean first, then replace/remeasure if still high loss. |
| Event 4, 59.909 km | Reflective, −40 dB; loss unresolved | Second connector 9 m after event 3; use short pulse from near end and power meter to measure combined insertion loss. |

Pulse spatial resolution is approximately cτ/(2n): 10 ns ≈ 1.02 m; 100 ns ≈ 10.21 m; 1 μs ≈ 102.1 m. Therefore, the 10 ns pulse can distinguish the two events 9 m apart in principle, but its 20 dB dynamic range is below the approximately one-way fiber-path loss plus 6 dB planning allowance (about 22.5 dB if all supplied path losses are included). The 100 ns setting provides 27 dB range but its 15 m ADZ exceeds the 9 m separation. A two-step test—long pulse for full reach and short pulse from the far end for localized resolution—is appropriate.

Round-trip delay t = 2nL/c. For n=1.4682, the approximate delays are: event 1 at 12.00 km: 117.5 μs; event 2 at 31.50 km: 308.5 μs; event 3 at 59.900 km: 586.9 μs; event 4 at 59.909 km: 587.0 μs. An index error of ±0.002 causes an absolute distance uncertainty of approximately L(0.002/1.4682): ±16 m at 12 km, ±43 m at 31.5 km and ±82 m near 60 km. This is a systematic location uncertainty; the relative 9 m separation is not scaled by the same absolute uncertainty if the same group index is used consistently.

Repair connector event 3/4 first due to its 1.20 dB loss and −22 dB reflectance, then replace event 2 with fusion splice. Clean connector end faces and avoid mating UPC and APC interfaces. LC is appropriate for a compact duplex patch panel; APC can reduce reflections if the complete interface is APC-compatible, but never mate APC and UPC directly. Verify by inspection/cleaning, insertion-loss measurement with a calibrated power meter/light source, and bidirectional OTDR.

## Design Decision 2

I recommend a long-pulse, high-dynamic-range OTDR trace for the full 60 km span and a short-pulse localized trace from the far end for the 9 m-spaced connector events. Repair event 3 first and event 2 next; confirm each repair using bidirectional OTDR and end-to-end power-meter loss.

# Stage 4 – Eye Diagram, BER and Quantum Limits (20 Marks)

## 4.1 Q-factor, BER and jitter

BER = 0.5 erfc(Q/√2). Using the supplied Q values, Channel 3 at Q=7.6 has BER approximately 1.5×10⁻¹⁴ (meets 10⁻¹²); Channel 5 at Q=4.7 has BER approximately 1.3×10⁻⁶ (fails by several orders of magnitude). The required Q for BER 10⁻¹² is stated as 7.03.

Total jitter at BER 10⁻¹² is TJ = DJ + 14.07 RJ. Channel 3: TJ = 0.18 + 14.07×0.012 = 0.349 UI, so estimated eye width = 1−TJ = 0.651 UI. Channel 5: TJ = 0.30 + 14.07×0.022 = 0.610 UI, eye width = 0.390 UI. Channel 5 has substantially less horizontal timing margin and a smaller vertical opening due to its low ER/noise.

| Measure | Channel 3 | Channel 5 | Acceptance |
| --- | --- | --- | --- |
| Q-factor | 7.6 | 4.7 | ≥7.03 |
| Approx. BER | 1.5×10⁻¹⁴ | 1.3×10⁻⁶ | ≤10⁻¹² |
| ER | 9.0 dB | 4.5 dB | ≥8.2 dB |
| DJ | 0.18 UI p-p | 0.30 UI p-p | Lower is better |
| RJ | 0.012 UI rms | 0.022 UI rms | Lower is better |
| TJ at BER 10⁻¹² | 0.349 UI | 0.610 UI | Lower is better |
| Eye width estimate | 0.651 UI | 0.390 UI | Larger is better |

Figure 2. Conceptual eye patterns illustrating a healthy open eye and a degraded partially closed eye;

Because both channels traverse the same fiber but Channel 3 has a healthy eye and Channel 5 has much worse ER and jitter, the evidence points to a channel-specific transmitter/driver, bias, connector path or receiver issue rather than fiber dispersion alone. Measure ER and waveform at the Channel 5 transmitter output and after the receiver; swap transceivers/ports if possible to isolate the cause.

## 4.2 Photon-counting and coherent detection limits

Photon energy at λ=1550 nm is Eγ=hc/λ≈1.282×10⁻¹⁹ J. For ideal direct detection, BER=0.5 exp(−N₁). Setting BER=10⁻¹² gives N₁=−ln(2×10⁻¹²)≈26.94 photons per ‘1’ bit. At 10 Gb/s this corresponds to P≈N₁EγR≈3.45×10⁻⁸ W = −44.6 dBm. The specified −24 dBm sensitivity is about 20.6 dB above this idealized limit.

For the supplied ideal PSK relations, solve 0.5 erfc(√(2N))=10⁻¹² for homodyne detection and 0.5 erfc(√N)=10⁻¹² for heterodyne detection. This gives approximately N≈12.7 photons/bit for homodyne and N≈25.4 photons/bit for heterodyne. At 10 Gb/s and 1550 nm these correspond to approximately −47.9 dBm and −44.9 dBm, respectively. These are idealized quantum limits, not practical receiver sensitivities; real systems incur implementation, shot-noise, thermal-noise, filtering and coding penalties.

## Design Decision 3

I recommend rejecting Channel 5 until ER is restored to at least 8.2 dB, Q rises to at least 7.03 and BER falls to 10⁻¹² or lower. Its low ER and high jitter explain a channel-specific failure more strongly than shared fiber impairment. The measured power margin must also be raised to at least 3 dB through repairs and end-to-end verification.

# Stage 5 – Integrated WDM Upgrade and Energy Strategy (20 Marks)

## 5.1 Grid and occupied spectrum

ITU-T G.694.1 uses a 193.1 THz anchor. For channel n=+3 on a 50 GHz grid, f=193.1 THz + 3×0.05 THz = 193.25 THz. Wavelength λ=c/f≈299,792,458/(193.25×10¹²)=1.5512 μm ≈1551.2 nm.

Option Q uses 32 GBd and roll-off α=0.1, so nominal occupied bandwidth per channel is Rs(1+α)=32×1.1=35.2 GHz. At 12.5 GHz flex-grid slot granularity, the required slot width is ceil(35.2/12.5)=3 slots = 37.5 GHz per channel. Eight channels occupy 8×37.5=300 GHz allocated spectrum.

| Option | Channels and format | Allocated spectrum | OSNR target | OSNR margin using 47 dB | Total module power | Energy/bit |
| --- | --- | --- | --- | --- | --- | --- |
| P | 16×100G DP-QPSK; 50 GHz slots | 800 GHz | 12.5 dB | 34.5 dB | 352 W | 220 pJ/bit |
| Q | 8×200G DP-16QAM; 3×12.5 GHz slots each | 300 GHz | 19.5 dB | 27.5 dB | 160 W | 100 pJ/bit |
| R | 4×400G Si-photonic; 75 GHz slots | 300 GHz | 26 dB | 21 dB | 60 W | 37.5 pJ/bit |
| Legacy reference | 160×10G SFP+ at 3.5 W | Not directly comparable | 20 dB | 27 dB by simplified formula | 560 W | 350 pJ/bit |

Energy per bit is module power divided by aggregate bit rate: P 352/1.6 Tb/s=220 pJ/bit; Q 160/1.6 Tb/s=100 pJ/bit; R 60/1.6 Tb/s=37.5 pJ/bit. The legacy comparison assumes 160 modules to deliver 1.6 Tb/s and excludes line-system overhead. Option R is most energy efficient and spectrally compact but requires 26 dB OSNR; Option Q provides a strong balance between spectrum, module power and OSNR margin while still meeting 1.6 Tb/s.

Figure 3. Calculated module power for the three 1.6-Tb/s upgrade options and a 10G SFP+ reference.

## 5.2 Proposed upgrade sequence

Repair the 1.20 dB connector and 0.62 dB mechanical splice; clean/inspect all end faces.

Re-measure insertion loss and OTDR from both directions; confirm total loss and at least 3 dB received-power margin.

Fix or replace the Channel 5 transceiver/driver and confirm ER ≥8.2 dB, Q ≥7.03 and BER ≤10⁻¹².

Measure OSNR and channel powers using the OSA; verify grid frequencies and spectral masks for the chosen transceivers.

Install Option Q as the recommended balanced upgrade candidate; compare Option R if its measured OSNR margin remains adequate with an engineering reserve.

Repeat end-to-end BER, Q, OSNR, power, eye/constellation, channel-spacing and thermal/power measurements after commissioning.

## Design Decision 4

I recommend Option Q (8×200G DP-16QAM) for the balanced deployment baseline. It provides 1.6 Tb/s in 300 GHz allocated spectrum with 160 W module power and 27.5 dB calculated OSNR margin under the provided OSNR relation. Option R is the energy leader at 37.5 pJ/bit but its 26 dB OSNR requirement leaves only 21 dB margin in the simplified model, so choose it only after measured OSNR, FEC performance and operational margin are verified.

# Stage 6 – Verification and Final Engineering Decision (10 Marks)

| Requirement | Target | Calculated / proposed evidence | Status |
| --- | --- | --- | --- |
| Total link loss | ≤21.25 dB | As-built 22.52 dB; repair events 2 and 3 then remeasure | Fail before repair |
| Received power | ≥−21 dBm for 3 dB margin vs −24 dBm | −21.52 dBm before repair/penalties | Fail by 0.52 dB raw |
| OSNR legacy | ≥20 dB / 0.1 nm | 47 dB from supplied formula | Pass in simplified model; verify by OSA |
| PMD | <10 ps | 0.775 ps estimated | Pass |
| Channel 3 | Q≥7.03, BER≤10⁻¹², ER≥8.2 dB | Q=7.6, BER≈1.5×10⁻¹⁴, ER=9 dB | Pass on stated metrics |
| Channel 5 | Q≥7.03, BER≤10⁻¹², ER≥8.2 dB | Q=4.7, BER≈1.3×10⁻⁶, ER=4.5 dB | Fail |
| Upgrade capacity | 1.6 Tb/s | Options P/Q/R each total 1.6 Tb/s | Capacity feasible |
| Upgrade energy | Minimize pJ/bit | P=220; Q=100; R=37.5 pJ/bit | R best energy; Q balanced |
| Spectrum | Use efficient grid | P=800 GHz; Q≈300 GHz; R=300 GHz | Q/R more efficient |
| Deployment | All criteria verified | Repair, test and then deploy Option Q candidate | Conditional |

Figure 4. Recommended workflow from baseline measurements through repair, channel verification and upgrade.

## Final Engineering Decision

DECISION: YES, WITH CONDITIONS. Do not deploy the current as-built system unchanged: its 22.52 dB total loss exceeds the 21.25 dB design budget, raw received-power margin is only 2.48 dB, and Channel 5 fails the ER, Q and BER criteria. Repair the poor connector and mechanical splice, correct the Channel 5 transmitter/receiver path, and verify the link using bidirectional OTDR, calibrated power-meter loss, OSA OSNR and BER/eye tests. After acceptance, Option Q is the recommended balanced 1.6-Tb/s upgrade candidate. Option R may be selected instead if measured OSNR and FEC results provide sufficient margin and the lowest energy per bit is the overriding objective.

# References

ITU-T Recommendation G.652, Characteristics of a single-mode optical fibre and cable; use the applicable G.652.D specification.

ITU-T Recommendation G.694.1, Spectral grids for WDM applications: DWDM frequency grid.

ITU-T optical fiber test and measurement recommendations relevant to OTDR, insertion loss and optical return loss; verify the applicable edition.

G. P. Agrawal, Fiber-Optic Communication Systems, 4th ed., Wiley, 2010.

G. P. Agrawal, Nonlinear Fiber Optics, 6th ed., Academic Press, 2019.

IEC 61280-4-2, Fibre-optic communication subsystem test procedures—installed single-mode fibre optic cable plant attenuation and optical return loss measurement (consult applicable edition).

Use the actual OTDR, OSA, power-meter and transceiver datasheets for instrument settings, calibration and final commissioning evidence.